JOB COSTING SHEET OVERVIEW

1. Weight Calculation
Weight (kg) = Length × Width × Thickness × Density
Steel Density = 7.85
Pipe Formula = (OD - Thickness) × Thickness × Length × 0.02466

2. Material Cost
Material Cost = Weight × Rate per kg

3. Fabrication Cost
Fabrication Cost = Weight × Rate per kg
OR
Fabrication Cost = Manhours × Hourly Rate

4. Manhours
Manhours = Weight × Factor
OR
Manhours = Quantity × Time per Unit

5. Welding
Weld Length = No. of Joints × Length per Joint
Welding Manhours = Weld Length × Time per meter
Welding Cost = Welding Manhours × Rate

6. Consumables
Consumables = Weld Length × Consumption Factor
Cost = Consumables × Rate
OR
Consumables = % of Fabrication Cost

7. Cutting / Machining
Cutting Cost = No. of Cuts × Rate
OR
Cutting Cost = Time × Rate

8. Surface Treatment
Area = Surface Area
Cost = Area × Rate per m²

9. Overheads
Overhead = Total Cost × %

10. Profit Margin
Selling Price = Total Cost × (1 + Margin %)

Important Notes:
- Avoid hardcoded values
- Maintain unit consistency (mm, m, kg)
- Avoid hidden multipliers
- Keep calculations modular and transparent